MARMARA UNIVERSITY

MECHANICAL ENGINEERING DEPARTMENT

ME7009 PROJECT

524618020 ÖZGÜRAZAD ÇELİK

Lecturer: Dr.Öğr.Üyesi UĞUR TÜMERDEM

ISTANBUL, 2022

Contents

System Linearization	2

System Initialization	4

Full-State Feedback	8

Pole Placement	10

Observer	14

LQR	20

KALMAN Filter	25

Model Linearizer	27

# System Linearization

# System Initialization

We have to control at the first controllability and observability of the system in order to apply control.

w=0.05

r0=5000

A=[0 1 0 0;

3*w^2 0 0 2*w*r0;

0 0 0 1;

0 (-2*w)/r0 0 0]

B=[0 0;

0 2;

0 0;

0 0]

C=eye(4,4);

C1=[0 0 0 0;

0 0 0 0;

0 0 1 0;

0 0 0 0];

D=zeros(4,2);

sys = ss(A,B,C,D)

[b,a] = ss2tf(A,B,C,D,2)

tf(sys)

% Co = ctrb(A,B)

Co = ctrb(sys)

unco = length(A) - rank(Co)

Ob= obsv(A,C)

% Number of unobservable states

unob = length(A)-rank(Ob)

w = 0.0500

r0 = 5000

A = 4×4

0    1.0000         0         0

0.0075         0         0  500.0000

0         0         0    1.0000

0   -0.0000         0         0

B = 4×2

0     0

0     2

0     0

0     0

sys =

A =

x1      x2      x3      x4

x1       0       1       0       0

x2  0.0075       0       0     500

x3       0       0       0       1

x4       0  -2e-05       0       0

B =

u1  u2

x1   0   0

x2   0   2

x3   0   0

x4   0   0

C =

x1  x2  x3  x4

y1   1   0   0   0

y2   0   1   0   0

y3   0   0   1   0

y4   0   0   0   1

D =

u1  u2

y1   0   0

y2   0   0

y3   0   0

y4   0   0

Continuous-time state-space model.

b = 4×5

0         0    2.0000         0         0

0    2.0000         0         0         0

0         0         0   -0.0000         0

0         0   -0.0000         0         0

a = 1×5

1.0000         0    0.0025         0         0

ans =

From input 1 to output...

1:  0

2:  0

3:  0

4:  0

From input 2 to output...

2 s

1:  --------------

s^3 + 0.0025 s

2 s^2

2:  --------------

s^3 + 0.0025 s

-4e-05 s

3:  ----------------

s^4 + 0.0025 s^2

-4e-05 s

4:  --------------

s^3 + 0.0025 s

Continuous-time transfer function.

Co = 4×8

0         0         0    2.0000         0         0         0   -0.0050

0    2.0000         0         0         0   -0.0050         0         0

0         0         0         0         0   -0.0000         0         0

0         0         0   -0.0000         0         0         0    0.0000

unco = 1

Ob = 16×4

1.0000         0         0         0

0    1.0000         0         0

0         0    1.0000         0

0         0         0    1.0000

0    1.0000         0         0

0.0075         0         0  500.0000

0         0         0    1.0000

0   -0.0000         0         0

0.0075         0         0  500.0000

0   -0.0025         0         0

unob = 0

We change the value of T due to uncontrollability.

B=[0 0;

0 2;

0 0;

-0.01 0]

Co = 4×8

0         0         0    2.0000   -5.0000         0         0   -0.0050

0    2.0000   -5.0000         0         0   -0.0050    0.0125         0

0         0   -0.0100         0         0   -0.0000    0.0001         0

-0.0100         0         0   -0.0000    0.0001         0         0    0.0000

unco = 0

Ob = 16×4

1.0000         0         0         0

0    1.0000         0         0

0         0    1.0000         0

0         0         0    1.0000

0    1.0000         0         0

0.0075         0         0  500.0000

0         0         0    1.0000

0   -0.0000         0         0

0.0075         0         0  500.0000

0   -0.0025         0         0

unob = 0

We provide controllability and observability with changing T.

We can find poles of the system find eigenvalues of matrix A.

% P = pole(sys);

Apoles = eig(A)

Apoles = 4×1 complex

0.0000 + 0.0000i

0.0000 + 0.0500i

0.0000 - 0.0500i

0.0000 + 0.0000i

# Full-State Feedback

% State-Feedback Gain Selection

p=[-2+i*2;-2-i*2;-1;-1]

K = place(A,B,p)

P = pole(sys)

plant = (A-B*K)

poles_cl = eig(plant)

p = 4×1 complex

-2.0000 + 2.0000i

-2.0000 - 2.0000i

-1.0000 + 0.0000i

-1.0000 + 0.0000i

K = 2×4

-800.0000 -799.9980 -400.0000 -500.0000

0.0037    0.5000   -0.5000  249.5000

P = 4×1 complex

0.0000 + 0.0000i

0.0000 + 0.0500i

0.0000 - 0.0500i

0.0000 + 0.0000i

plant = 4×4

0    1.0000         0         0

0.0000   -1.0000    1.0000    1.0000

0         0         0    1.0000

-8.0000   -8.0000   -4.0000   -5.0000

poles_cl = 4×1 complex

-2.0000 + 2.0000i

-2.0000 - 2.0000i

-1.0000 + 0.0000i

-1.0000 + 0.0000i

# Pole Placement

We use 3 times faster roots

des_poles = (min(real(poles_cl))*3)-(1:4);

des_poles

K = place(A,B,des_poles)

step(sys);

des_poles = 1×4

-7.0000   -8.0000   -9.0000  -10.0000

K = 2×4

103 ×

0.7791    0.0918   -6.9333   -1.6740

0.0368    0.0086   -0.0043    0.2495

As we can see from plot, our settling time and overshoot percentage is decreased.

sys2=ss(A-B*K,B,C,D);

tf(sys2)

step(sys2)

Kdc=dcgain(sys2);

ans =

From input 1 to output...

-0.01013 s - 0.08653

1:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

-0.01013 s^2 - 0.08653 s - 2.918e-17

2:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

-0.01 s^2 - 0.1726 s - 0.7367

3:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

-0.01 s^3 - 0.1726 s^2 - 0.7367 s

4:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

From input 2 to output...

2 s^2 + 33.48 s + 138.7

1:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

2 s^3 + 33.48 s^2 + 138.7 s

2:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

1.837 s + 15.58

3:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

1.837 s^2 + 15.58 s + 3.684e-15

4:  --------------------------------------

s^4 + 34 s^3 + 431 s^2 + 2414 s + 5040

Continuous-time transfer function.

# Observer

L=place(A.',C.',des_poles)

obsplant=(A-L*C)

poles_obs=eig(obsplant)

des_poles1 = (min(real(poles_obs))*3)-(1:4); %This is better

des_poles1

L = 4×4

7.0000    0.0075         0         0

1.0000    8.0000         0   -0.0000

0         0    9.0000         0

0  500.0000    1.0000   10.0000

obsplant = 4×4

-7.0000    0.9925         0         0

-0.9925   -8.0000         0  500.0000

0         0   -9.0000    1.0000

0 -500.0000   -1.0000  -10.0000

poles_obs = 4×1 complex

102 ×

-0.0900 + 5.0000i

-0.0900 - 5.0000i

-0.0700 + 0.0000i

-0.0900 + 0.0000i

des_poles1 = 1×4

-28.0000  -29.0000  -30.0000  -31.0000

Aobs = [A-L*C]

Bobs = [B L]

Cobs=[0 0 0 0;

0 0 0 0;

0 0 1 0;

0 0 0 0];

Cobs1=[1.1 0 0 0;

0 1 0 0;

0 0 1 0;

0 0 0 1];

Dobs = zeros(4,6);

Aobs = 4×4

-7.0000    0.9925         0         0

-0.9925   -8.0000         0  500.0000

0         0   -9.0000    1.0000

0 -500.0000   -1.0000  -10.0000

Bobs = 4×6

0         0    7.0000    0.0075         0         0

0    2.0000    1.0000    8.0000         0   -0.0000

0         0         0         0    9.0000         0

-0.0100         0         0  500.0000    1.0000   10.0000

In first attempt we tried slower roots and then we increased its speed. Therefore we observe close performance that we measure before.

Using observer with faster roots Show us same result with greater magnitude than Full-State Feedback controller.

Only the member third columns of third row is equal to 1. So only this point permit transmitting the signal.

Cobs=[0 0 0 0;

0 0 0 0;

0 0 1 0;

0 0 0 0];

This is plot of r measure when the  only the angle theta is measured as output

# LQR

%%%LQR 1%%%

Q =[1 0 0 0;

0 1 0 0;

0 0 1 0;

0 0 0 1]

R =eye(2)

N=0;

[Klqr,S,E]=lqr(A,B,Q,R,N)

Nbar= -pinv(C*inv(A-B*Klqr)*B);

G=Nbar;

plant_lqr = (A-B*Klqr)

poles_lqr = eig(plant_lqr)

Q = 4×4

1     0     0     0

0     1     0     0

0     0     1     0

0     0     0     1

R = 2×2

1     0

0     1

Klqr = 2×4

-0.4230   -0.7254   -0.9061 -227.3233

0.9099    1.1750   -0.4230  145.0888

S = 4×4

104 ×

0.0001    0.0000    0.0000    0.0042

0.0000    0.0001   -0.0000    0.0073

0.0000   -0.0000    0.0250    0.0091

0.0042    0.0073    0.0091    2.2732

E = 4×1 complex

-0.0040 + 0.0000i

-0.9789 + 0.0000i

-1.8202 + 1.3397i

-1.8202 - 1.3397i

plant_lqr = 4×4

0    1.0000         0         0

-1.8123   -2.3501    0.8460  209.8224

0         0         0    1.0000

-0.0042   -0.0073   -0.0091   -2.2732

poles_lqr = 4×1 complex

-1.8202 + 1.3397i

-1.8202 - 1.3397i

-0.9789 + 0.0000i

-0.0040 + 0.0000i

In first LQR model we are used Identity matrix for Q.

But we need to observe r values of system. Significance of this of this value more important for us. Because we are trying to control its Radius to mass center.

Therefore we are going to increase fisrt member of Q matrix.

%%%LQR 2%%%

Q2 =[100 0 0 0;

0 1 0 0;

0 0 1 0;

0 0 0 1]

R2 =eye(2)

N2=0;

[Klqr2,S2,E2]=lqr(A,B,Q2,R2,N2)

Nbar2= -pinv(C*inv(A-B*Klqr2)*B);

G=Nbar;

plant_lqr2 = (A-B*Klqr2)

poles_lqr2 = eig(plant_lqr2)

Q2 = 4×4

100     0     0     0

0     1     0     0

0     0     1     0

0     0     0     1

R2 = 2×2

1     0

0     1

Klqr2 = 2×4

-1.4855   -1.0648   -0.9889 -247.6417

9.8928    3.1233   -0.1485  212.9547

S2 = 4×4

104 ×

0.0032    0.0005    0.0001    0.0149

0.0005    0.0002   -0.0000    0.0106

0.0001   -0.0000    0.0250    0.0099

0.0149    0.0106    0.0099    2.4764

E2 = 4×1 complex

-0.0040 + 0.0000i

-2.3972 + 0.0000i

-3.1609 + 3.2965i

-3.1609 - 3.2965i

plant_lqr2 = 4×4

0    1.0000         0         0

-19.7781   -6.2465    0.2971   74.0905

0         0         0    1.0000

-0.0149   -0.0107   -0.0099   -2.4764

poles_lqr2 = 4×1 complex

-3.1609 + 3.2965i

-3.1609 - 3.2965i

-2.3972 + 0.0000i

-0.0040 + 0.0000i

We can see that some poles are getting faster.

%%%LQR 3%%%

Q3 =[1000 0 0 0;

0 1 0 0;

0 0 100 0;

0 0 0 1]

R3 =eye(2)

N3=0;

[Klqr3,S3,E3]=lqr(A,B,Q3,R3,N3)

Nbar3= -pinv(C*inv(A-B*Klqr3)*B);

G=Nbar;

plant_lqr3 = (A-B*Klqr3)

poles_lqr3 = eig(plant_lqr3)

Q3 = 4×4

1000           0           0           0

0           1           0           0

0           0         100           0

0           0           0           1

R3 = 2×2

1     0

0     1

Klqr3 = 2×4

-2.0614   -1.1686   -9.9787 -253.4403

31.5593    5.5847   -0.6519  233.7282

S3 = 4×4

104 ×

0.0179    0.0016    0.0008    0.0206

0.0016    0.0003   -0.0000    0.0117

0.0008   -0.0000    0.2540    0.0998

0.0206    0.0117    0.0998    2.5344

E3 = 4×1 complex

-0.0400 + 0.0000i

-2.4877 + 0.0000i

-5.5880 + 5.6854i

-5.5880 - 5.6854i

plant_lqr3 = 4×4

0    1.0000         0         0

-63.1110  -11.1694    1.3037   32.5436

0         0         0    1.0000

-0.0206   -0.0117   -0.0998   -2.5344

poles_lqr3 = 4×1 complex

-5.5880 + 5.6854i

-5.5880 - 5.6854i

-2.4877 + 0.0000i

-0.0400 + 0.0000i

We can understand that the poles placed upside are decreasing faster.

%%%LQR 4%%%

Q4 =[1000 0 0 0;

0 1 0 0;

0 0 100 0;

0 0 0 1]

R4 =[100 0;

0 10]

N4=0;

[Klqr4,S4,E4]=lqr(A,B,Q4,R4,N4)

Nbar4= -pinv(C*inv(A-B*Klqr4)*B);

G=Nbar;

plant_lqr4 = (A-B*Klqr4)

poles_lqr4 = eig(plant_lqr4)

Q4 = 4×4

1000           0           0           0

0           1           0           0

0           0         100           0

0           0           0           1

R4 = 2×2

100     0

0    10

Klqr4 = 2×4

-0.0799   -0.1219   -0.9997  -80.2874

10.0006    3.1539   -0.0799  243.7770

S4 = 4×4

105 ×

0.0032    0.0005    0.0001    0.0080

0.0005    0.0002   -0.0000    0.0122

0.0001   -0.0000    0.0803    0.1000

0.0080    0.0122    0.1000    8.0287

E4 = 4×1 complex

-0.0127 + 0.0000i

-0.7901 + 0.0000i

-3.1540 + 3.1721i

-3.1540 - 3.1721i

plant_lqr4 = 4×4

0    1.0000         0         0

-19.9936   -6.3078    0.1598   12.4459

0         0         0    1.0000

-0.0008   -0.0012   -0.0100   -0.8029

poles_lqr4 = 4×1 complex

-3.1540 + 3.1721i

-3.1540 - 3.1721i

-0.7901 + 0.0000i

-0.0127 + 0.0000i

When we increase value of R matrix, we observe that speed of poles decreased to previous state

# KALMAN Filter

In Kalman Filter we used Q2 and N2 that we used before for LQR.

# Model Linearizer